Adilet Kuroda

Ritika Raju

CS 367 - Recitation #12				    						Architecture

Today's Goals: Get comfortable with logic gates, digital logic, and sequential design.

Work in groups of 2-3 students. Every group will turn in what they've got at the end of recitation on paper, with all the members’ names written down.

Grading is based on participation. Get as much done as you can. You will also be given feedback in the form of a 'score' (1-3) and possibly some comments. This doesn’t affect your grade – it is solely for feedback. A score of 3 means everything looks great.   A score of two indicates some minor problems.  And a score of one indicates that there were some major issues. If you get a 1, don't panic - go see your prof or a GTA to get more extensive feedback.

Digital Logic

In class, we introduced four digital logic gates: AND, OR, NOT, and XOR.

| For each gate, the inputs and output are each 1-bit values. These gates can have any number of inputs (2, 3, or 4 inputs are all fine), but only one output. |  |  |  |
| --- | --- | --- | --- |
| AND | OR | NOT | XOR |

For 1-bit operations, we can use the following symbols to refer to these gates in an expression:

| AND | OR | NOT | XOR |
| --- | --- | --- | --- |
| && | ¦¦ | ! or ~ | ^ |

Task 1:    Select from the list of expressions, which matches each of these diagrams:

|  | Expression: - a ¦¦ (~b && c) |
| --- | --- |

Write an expression for the following circuit:

|  | Expression: (a && ~b) ¦¦ (~a && c) |
| --- | --- |

Digital Logic

Task 2:  In class, we introduced four digital logic gates: AND, OR, NOT, and XOR.

In this exercise, fill in the area in the dashed lines with a circuit drawing for each of the five areas to implement a very simple Arithmetic Logic Unit (ALU).  You can use these four gates:

| For each gate, the inputs and output are each 1-bit values. You can use these gates with any number of inputs (2, 3, or 4 inputs are all fine), but only one output. |  |  |  |
| --- | --- | --- | --- |
| AND | OR | NOT | XOR |

Complete the implementation of the below ALU to perform the following functions based on s inputs.

| When s0 and s1 are… | s0 = 0 and s1 = 0 | s0 = 1 and s1 = 0 | s0 = 0 and s1 = 1 | s0 = 1 and s1 = 1 |
| --- | --- | --- | --- | --- |
| Perform the following… | a && b | ~ (a ¦¦ b) | ~a ^ b | a != b |

Pipeline Hazards

Task3:  In class, we did a brief introduction into the idea of hazards in the Pipelines.  A Data Hazard occurs when data from one instruction is needed for a future instruction, but that data hasn’t finished writing back.

Discuss the following code within your group and try and find any such hazards in each block of code.   If you identify any, rewrite the code using the nop instruction to very precisely delay the operation so the data is ready for it exactly when it is needed.  (hint: W has to complete before the data is in the register)

Refresher:  There are 5 stages for each execution (Fetch, Decode, Execute, Memory, Writeback)

Code 1:

1 	movq $10, %rdx

2 	incq %rcx

3 	incq %rcx

4 	addq %rdx, %r8

5   	movq $0, %rax

Which of the above instructions will face pipeline data hazards? (Data not available when needed)

incq %rcx

incq %rcx

Which stage will instruction 2 be in when instruction 3 needs RCX?

Execute

Rewrite the above code to include enough NOP instructions to make sure no hazards exist.

- movq $10, %rdx

- addq $2, %rcx

4  NOP

5. NOP

6. NOP

7 incq %rcx

8 addq %rdx, %r8

9 movq $0, %rax

10